Creation of Algorithms and Software For Use With a “Foosbot”

By: Michael Meadows

January 2008

Computer Systems Lab, 2008-2009

## 1-Abstract:

## The purpose of the project is to create a functional AI to be physically integrated with a full sized foosball table. The project will involve two groups members that will divide responsibilities between the physical automation and the “intelligence” behind the automation. This paper will deal with the AI aspect of the project.

The Project will be dealt with in three stages. The tracking will developed first. The defense and physical movement will come into play second. The offensive algorithms will be the third and final aspect of the project to be implemented. The idea behind all aspects of the project is to create an AI that will grow in skill the more it plays by analyzing past experiences. Some specific scenarios will be hard coded into the bot, but that is not the purpose of the project.

## Introduction

Simple scenarios such as tic-tac-toe can be hard coded into systems because of the limited supply of possibilities. In more complex games such as chess the number of options grows extremely quickly and the ability to code for specific scenarios declines, however the computer can still have code written for types of scenarios. In the game of foosball we run into a different problem. Not all movements work in the same way. Just because the bot hits the ball at a certain angle at a certain time does not mean it will behave exactly as it did the time before. The reason for this is that there are far to many tiny errors we can not account for. the ball is not shaped perfectly nor is the tables surface. Due to imperfections in our control methods we are also unable to perfectly replicate a shot from instance to instance. In order to meet the challenges of an environment the is not constant we seek to program an AI that will constantly adapt to the imperfect and changing environment.

# Statement of Problem

In order to allow AI's to continue to expand there areas of expertise we need to create a more effective method of programming them. Human beings are not coded from birth for every challenge that they will face, instead they are given a set of tools to adapt to new challenges. To replicate that effect we will strive to create a self taught AI only given guidelines for how it should develop.

# Statement again of the Purpose/Goal (from the Abstract)

The problem set before the group is an integration of motion tracking with predictive algorithms and using these variables, to create a useful output to manifest in the movements of pneumatic reel control devices. The topic has many potential beneficial outcomes. Assuming that the end product is competitive and costs less than 32,00 dollars there is potential to take the project to the market. Should the project fail to play on a highly competitive level then it shall serve a demonstration to future students and may serve as an educational resource by providing insight as to the integration of our three components.

The purpose of the project is to create a functional AI to be physically integrated with a full sized foosball table. The project will involve two groups members that will divide responsibilities between the physical automation and the “intelligence” behind the automation. This paper will deal with the AI aspect of the project.

Background

# The concept of self teaching AI is definetly not new. It is not even new in the application of a "foosbot". German scientists have created a commercial table for sale, and several universities have created full projects on the creation of these tables, though most have met limited or no success.

# likely, you're not the first to be working on this idea.) Discuss how these provide a basis for your project.    Be

# clear, concise, and as thorough as possible in summary of this previous work and background information.

# Literature Review, background of algorithms and theory

Because the two purposes of this section are to prepare the reader to understand your project and to highlight your expertise on the topic, you should review five or more pertinent sources to provide background information on the topic. You should also briefly define terms and describe specific software and algorithms and theory used in the study.

# All information in this section should be cited.

Development Sections (Your actual program/project, what you actually did this year)

﻿

Creating a functional program to manage the movements of Foosball robot is a multistage process with several large obstacles. The fundamental difference between the play style of the robot and the human, is that while the robot may have better reaction time (.05 seconds and up in our case as compared to a humans .1), the human has a much better innate ability to recognize patterns and predict actions then does a computer.

To counter this discrepancy the team will attempt to improve on two fronts at the same time. The team will attempt to increase the frame rate (Reaction time of the bot) as well as the precision and speed of movement of the mechanical system. Secondly the team will attempt to improve the computers ability to predict ball's movement to compensate for imperfect reaction time.

In order for the bot's AI to function it will need a method to determine the balls position in real-time. To solve this issue the group determined to use software from Processing.org and a Video Digitizer. This software working together converts a video stream into a 2-D array the values of which will contain the colors of the pixels. Using a preselected ball color that is significantly different from the rest of the objects on the table, we should be able to determine the balls position in real time by examining the values in the video array. We require and expect to have at least .35in resolution from the camera.

Once the balls position on the table can be determined we can begin the construction of the defensive algorithm. At every frame the balls position will be recorded, the position will also be recorded in the subsequent frame. The first and second frame position will be used as keys in a map where the contained value will be the coordinates where the ball should cross the subsequent reels. The contained value will be determined simply by an average of past experiences. The defense will not so much anticipate the ball movement as it will remember previous scenarios that were identical. Over time the program will create a map of nearly all unique ball paths (Not all paths will be recorded, only the ones that are significantly different from previous paths.)

The offensive algorithm will come in second and will utilize a different approach. The offense will record 50 algorithms to describe 50 different shots. Every time the program takes a shot that is not described by one of the 50 algorithms(it will shoot randomly with only a few guidelines about 10% of the time), it will create a new algorithm to describe that shot (initial position of ball and foosmen (Objects along the reels), as well as amount of power). As each algorithm is used it will be awarded points for weather or not it went into the goal, or if not then how close it came to going in. If the newly generated shot's point score is higher than any of the previous 50 shots, the lowest rated algorithm is forgotten and the new shot is adopted into the shot table. Over time, and by taking new shots randomly the machine should acquire 50 accurate and hard to block shots.

## Developmental Procedures

Explain all steps clearly so the method of production could be repeated by others.    Include diagrams and working drawings when appropriate.    The completed product is tested for performance under use (QA), and then analyzed and re-designed if necessary.

Quality Assessment (QA), Validation of your program's performance

Results, Discussion, Conclusion, Recommendations (“End matter” for your paper)

##### Discussion

###### Conclusion

# Literature Cited

# Correct format, no web-only citations

# Acknowledgments

# Appendix

# Sources (eight minimum, hard copy attached to paper)

# Format

Correct APA format throughout

Proper paragraphing, spelling, and grammar

Text body is 12 pages or less/ Total (i.e., with Appendix) is 20 pages or less/double-spaced

All diagrams appropriately labeled (and cited, if necessary)